Table S1 | Parameter bounds and bounded decoding map for MXene EIS inference. Parameter-domain constraints (lower/upper bounds) and the corresponding decoding rules used consistently across the classical NLLS baseline, the continuous VQE/VQA branch, and the discrete QUBO/QAOA branch for the 7-parameter circuit θ={Rs,L,Rct,Q1,α1,Q2,α2}. The table specifies each parameter’s physical meaning and units and indicates whether the optimizer operates in a log-scaled or linear coordinate, with sigmoid-bounded mapping where applicable to guarantee feasibility (e.g., R>0, 0<α≤1) during optimization and decoding.

| Parameter | Description | Units | Lower Bound | Upper Bound | Encoding Strategy |
| --- | --- | --- | --- | --- | --- |
| L | Inductance | H | 1.88×10-8 | 1.88×10-6 | Log-scale, sigmoid-bounded |
| Rs | Series resistance | Ω | 0.0873 | 8.73 | Log-scale, sigmoid-bounded |
| Rct | Charge-transfer resistance | Ω | 1.5 | 150 | Log-scale, sigmoid-bounded |
| Q1 | CPE magnitude (branch 1) | S·sα1 | 2.62×10-4 | 0.0262 | Log-scale |
| α1 | CPE exponent (branch 1) | – | 0.3 | 0.99 | Linear, sigmoid |
| Q2 | CPE magnitude (branch 2) | S·sα2 | 2.12×10-4 | 0.0212 | Log-scale |
| α2 | CPE exponent (branch 2) | – | 0.3 | 0.99 | Linear |